A Mini Project Report

## An Intelligent System for Environment Quality Remote Monitoring System

##### Submitted to the

GURUNANAK INSTITUTIONS OF TECHNICAL CAMPUS IBRAHIMPATNAM, HYDERABAD

In partial fulfillment of the requirement for the award of the degree of

BACHELOR OF TECHNOLOGY IN

ELECTRONICS AND COMMUNICATION ENGINEERING

##### Submitted by

S. MAHESH REDDY  (18WJ5A045)

M.SHIVA SAI REDDY  (18WJ5A0472)

U. VISHAL GOUD  (18WJ5A045)

Under the Guidance of

MS.MANPREET KAUR

Assistant Professor

SCHOOL OF ENGINEERING & TECHNOLOGY GURU NANAK INSTITUTIONS TECHNICAL CAMPUS

(Affiliated to JNTU, Hyderabad, Approved by AICTE) Ibrahimpatanam, Ranga Reddy District -501506 Telangana, India.

2020-2021

|  | Guru Nanak Institutions Technical Campus School of Engineering & Technology Department of Electronics and Communication Engineering |
| --- | --- |

## CERTIFICATE

This is to certify that the Mini project entitled “An Intelligent System for Environment Quality Remote Monitoring System” is being presented with report by S.MAHESH REDDY (18WJ5A0459), M.SHIVA SAI REDDY (18WJ5A0472), U.VISHAL GOUD(18WJ5A0465), in partial

fulfillment for the award of Degree of Bachelor of Technology in Electronics and Communication Engineering to the Guru Nanak Institutions Technical Campus, Ibrahimpatnam, Hyderabad.

##### INTERNAL GUIDE	PROJECT CO-ORDINATOR

Ms. Manpreet Kaur	Dr. Harpreet Kaur

Assistant Professor	Associate Professor

##### HEAD OF DEPARTMENT	EXTERNAL EXAMINER

Dr. S.P.Yadav

Professor & HOD-ECE

## ACKNOWLEDGEMENTS

We would like to express our sincere gratitude to our guide Ms. Manpreet Kaur, Assistant Professor, Department of Electronics and Communication Engineering, for her excellent guidance and invaluable support, which helped us accomplish our Bachelor degree and prepared us to achieve more life goals in the future. Her total support of our dissertation and countless contributions to our technical and professional development made for a truly enjoyable and fruitful experience. Special thanks are dedicated for the discussions we had on almost every working day during our project period and for reviewing our dissertation.

We are very much grateful to Dr. Harpreet Kaur, Associate Professor, Project Co- ordinator, ECE, GNITC, Hyderabad, who has not only shown utmost patience, but was fertile in suggestions, vigilant in directions of error and has been infinitely helpful.

We are extremely grateful to Dr. S.P. Yadav, Professor & HOD of ECE, GNITC, Hyderabad for being so thoughtful and helpful with truly valuable insights and guidance during our project.

We wish to express our deepest gratitude and thanks to Dr. Ravi Kishore Singh, Associate Director, GNITC for his constant support and encouragement and for providing us all the facilities in the college during our project work.

Our sincere thanks to all our faculties, administrative staff and management of GNITC, who helped us in completing the mini project.

On a more personal note we thank our beloved parents and friends for their moral support during the course of our project.

### TABLE OF CONTENTS

| INDEX |  |  |  |  |
| --- | --- | --- | --- | --- |
| Chapter No. | TOPIC |  | Page no |
| 1 1.1 1.2 1.3 | INTRODUCTION General Existing System Proposed System |  | 1 1 2 2 |  |
| 2 2.1 2.2 2.3 2.3.1 2.3.2 2.3.3 2.3.4 2.4 2.4.1 2.4.2 2.4.3 2.4.4 2.4.5 2.4.6 2.4.7 2.4.8 2.5 2.5.1 2.5.2 2.5.3 2.6 2.6.1 2.6.2 2.7 2.8 2.8.1 2.8.2 2.9 2.10 2.11 2.11.1 | Project Description General Introduction To Embedded System Block Diagram Modules Power Supply Transformer Rectifier Voltage Regulator MICROCONTROLLER Microcontroller in embedded system ###### Basic structure of a microcontroller i/o subsystem Registers ###### Memory Features ### Pin Description Ports Liquid Crystal Display (Lcd) Introduction Pin Diagram List Of Commands Temperature sensor Specifications Applications Interfacing circuit(LM358) Humidity sensor Features Applications DRIVER CIRCUIT (L293D) DC MOTOR Global system for mobile Features |  | 3 4 5 10 10 11 12 13 |  |
| 3 3.1 3.2 3.3 3.4 | SOFTWARE SPECIFICATIONS KEIL SOFTWARE Introduction # µVision3 Overview STEPS |
| 4 4.1 | IMPLEMENTATION Schematic Design |
| 5 5.1 | SIMULATION AND DESIGN Output Screen Shots |
| 6 | CONCLUSION |
| 7 | REFERENCES |

References

Annexure: Source Code

| Figure no. | Name of the Figure | Page no. |
| --- | --- | --- |
| Fig 3.1 | Block Diagram | 7 |
| Fig 3.2 | ESP8266 | 8 |
| Fig 3.3 | Block diagram of ESP8266 | 9 |
| Fig 3.4 | NodeMCU | 10 |
| Fig 3.5 | Pin diagram of NodeMCU | 11 |
| Fig 3.6 | GPIO pins | 12 |
| Fig 3.7 | Arduino UNO | 14 |
| Fig 3.8 | Arduino UNO Pin Configuration | 17 |
| Fig 3.9 | Arduino NANO | 18 |
| Fig 3.10 | Arduino NANO Pin Configuration | 20 |
| Fig 3.11 | DC Motors | 21 |
| Fig 3.12 | L293D Motor Driver | 22 |
| Fig 3.13 Fig 3.14 Fig 3.15 Fig 3.16 Fig 3.17 Fig 3.18 Fig 3.19 Fig 3.20 Fig 3.21 Fig 3.22 Fig 3.23 Fig 3.24 Fig 3.25 Fig 3.26 | L293D Pin Configuration L293D Circuit Diagram Flame Sensor ADXL335 ADXL335 Circuit Diagram ADXL335 Pin Configuration ADXL335 Working Principle Relay Module Motor Pump Bluetooth Module – HC05 Bluetooth Module Pin Configuration Chassis Breadboard Jump Wires | 23 24 26 27 28 30 31 32 33 33 34 35 35 36 |
| Fig 4.1 | Flow Chart | 38 |
| Fig 4.2 | Arduino IDE | 39 |
| Fig 4.3 | Arduino IDE Set-Up | 40 |
| Fig 5.1 Fig 5.2 | Transmitter section Schematic diagram Receiver section Schematic diagram | 52 53 |

| Table no. | Name of the table | Page no. |
| --- | --- | --- |
| Table 3.1 | Specifications of NodeMCU | 13 |
| Table 3.2 | Specifications of Arduino UNO | 15 |
| Table 3.3 | Specifications of Arduino NANO | 18 |
| Table 3.4 Table 3.5 | Pin Configuration of Arduino UNO Pin Configuration of L293D | 19 23 |
| Table 3.6 Table 3.7 | Pin Configuration of Flame Sensor Specifications of ADXL335 | 26 28 |

| LCD | Liquid Crystal Display |
| --- | --- |

## ABSTRACT

The weather report is based on the multi parameters such as temperature, moisture level etc., the temperature and moisture level can be measured by the temperature sensor and moisture sensor. These sensors are connected with the robot. The robot can move from one place to another place. So the multi parameters value can measure from multi places. Then the report will transfer to the station by using GSM.

CHAPTER 1

INTRODUCTION

1.1GENERAL

The weather report is based on the multi parameters such as temperature, moisture level etc., the temperature and moisture level can be measured by the temperature sensor and moisture sensor. These sensors are connected with the robot. The robot can move from one place to another place. So the multi parameters value can measure from multi places. Then the report will transfer to the station by using GSM.

1.2 EXISTING SYSTEM:

In the earlier system the Green house report based on one place only. We can’t measure the parameter value in the multiple areas at the same time. So we go for a new system to avoid this problem.

1.3 PROPOSED SYSTEM

In this system the robot can measure the temperature and moisture values in the multiple areas by robot movements. The GSM modem will connect to the robot this GSM modem will act as a receiver. TheL293D are used to control the robot movements. After receive the sensors abnormal status the robot will send a message and move to another place and sense the values and transmit the values to the user. All the status will be displayed by the LCD.

CHAPTER 2

PROJECT DESCRIPTION

2.1 GENERAL INTRODUCTION TO EMBEDDED SYSTEM

Embedded systems are designed to do some specific task rather than be a general purpose computer for multiple tasks. Some also have real time performance constraints that must met, for reason such as safety and usability; others may have low or no performance requirements, allowing the system hardware to be simplified to reduce costs.

An embedded system is not always a separate block very often it is physically built in to the device it is controlling. The software written for embedded systems is often called firmware, and is stored in read only memory or flash convector chips rather than a disk drive. It often runs with limited computer hardware resources: small or no keyboard, screen and little memory.

To perform any application in the embedded system we require microprocessor and microcontroller. In the microprocessor an external memory is connected which increases the size of the microprocessor and multiple operations are being performed by the microprocessor but whereas in the microprocessor the memory is inbuilt and also we can use this controller only for the specific applications where the speed is increased so most probably microcontrollers are used in the different applications in the embedded systems rather than microprocessor.

An embedded system can be defined as a computing device that does a specific focused job. Appliances such as the air-conditioner, VCD player, DVD player, printer, fax machine, mobile phone etc. are examples of embedded systems. Each of these appliances will have a processor and special hardware to meet the specific requirement of the application along with the embedded software that is executed by the processor for meeting that specific requirement. The embedded software is also called “firm ware”. The desktop/laptop computer is a general purpose computer. You can use it for a variety of applications such as playing games, word processing, accounting, software development and so on. In contrast, the software in the embedded systems is always fixed listed below:

.

Embedded systems do a very specific task; they cannot be programmed to do different things. . Embedded systems have very limited resources, particularly the memory. Generally, they do not have secondary storage devices such as the CDROM or the floppy disk. Embedded systems have to work against some deadlines. A specific job has to be completed within a specific time. In some embedded systems, called real-time systems, the deadlines are stringent. Missing a deadline may cause a catastrophe-loss of life or damage to property. Embedded systems are constrained for power. As many embedded systems operate through a battery, the power consumption has to be very low.

· Some embedded systems have to operate in extreme environmental conditions such as very high temperatures and humidity.

Application Areas

Nearly 99 per cent of the processors manufactured end up in embedded systems. The embedded system market is one of the highest growth areas as these systems are used in very market segment- consumer electronics, office automation, industrial automation, biomedical engineering, wireless communication, data communication, telecommunications, transportation, military and so on.

Consumer appliances:

At home we use a number of embedded systems which include digital camera, digital diary, DVD player, electronic toys, microwave oven, remote controls for TV and air-conditioner, VCO player, video game consoles, video recorders etc. Today’s high-tech car has about 20 embedded systems for transmission control, engine spark control, air-conditioning, navigation etc. Even wristwatches are now becoming embedded systems. The palmtops are powerful embedded systems using which we can carry out many general-purpose tasks such as playing games and word processing.

Office Automation:

The office automation products using embedded systems are copying machine, fax machine, key telephone, modem, printer, scanner etc.

Industrial Automation:

Today a lot of industries use embedded systems for process control. These include pharmaceutical, cement, sugar, oil exploration, nuclear energy, electricity generation and transmission. The embedded systems for industrial use are designed to carry out specific tasks such as monitoring the temperature, pressure, humidity, voltage, current etc., and then take appropriate action based on the monitored levels to control other devices or to send information to a centralized monitoring station. In hazardous industrial environment, where human presence has to be avoided, robots are used, which are programmed to do specific jobs. The robots are now becoming very powerful and carry out many interesting and complicated tasks such as hardware assembly.

Medical Electronics:

Almost every medical equipment in the hospital is an embedded system. Theseequipment include diagnostic aids such as ECG, EEG, blood pressure measuring devices, X-ray scanners; equipment used in blood analysis, radiation, colonoscopy, endoscopy etc. Developments in medical electronics have paved way for more accurate diagnosis of diseases.

Computer Networking:

Computer networking products such as bridges, routers, Integrated Services Digital Networks (ISDN), Asynchronous Transfer Mode (ATM), X.25 and frame relay switches are embedded systems which implement the necessary data communication protocols. For example, a router interconnects two networks. The two networks may be running different protocol stacks. The router’s function is to obtain the data packets from incoming pores, analyze the packets and send them towards the destination after doing necessary protocol conversion. Most networking equipment’s, other than the end systems (desktop computers) we use to access the networks, are embedded systems.

Telecommunications:

In the field of telecommunications, the embedded systems can be categorized as subscriber terminals and network equipment. The subscriber terminals such as key telephones, ISDN phones, terminal adapters, web cameras are embedded systems. The network equipment includes multiplexers, multiple access systems, Packet Assemblers Dissemblers (PADs), sate11ite modems etc. IP phone, IP gateway, IP gatekeeper etc. are the latest embedded systems that provide very low-cost voice communication over the Internet.

Wireless Technologies:

Advances in mobile communications are paving way for many interesting applications using embedded systems. The mobile phone is one of the marvels of the last decade of the 20’h century. It is a very powerful embedded system that provides voice communication while we are on the move. The Personal Digital Assistants and the palmtops can now be used to access multimedia service over  the Internet. Mobile communication infrastructure such as base station controllers, mobile switching centers are also powerful embedded systems.

Insemination:

Testing and measurement are the fundamental requirements in all scientific and engineering activities. The measuring equipment we use in laboratories to measure parameters such as weight, temperature, pressure, humidity, voltage, current etc. are all embedded systems. Test equipment such as oscilloscope, spectrum analyzer, logic analyzer, protocol analyzer, radio communication test set etc. are embedded systems built around powerful processors. Thank to miniaturization, the test and measuring equipment are now  becoming portable facilitating easy testing and measurement in the field by field-personnel.

Security:

Security of persons and information has always been a major issue. We need to protect our homes and offices; and also the information we transmit and store. Developing embedded systems for security applications is one of the most lucrative businesses nowadays. Security devices at homes, offices, airports etc. for authentication and verification are embedded systems. Encryption devices are nearly 99 per cent of the processors that are manufactured end up in~ embedded systems. Embedded systems find applications in every industrial segment- consumer electronics, transportation, avionics, biomedical engineering, manufacturing, process control and industrial automation, data communication, telecommunication, defense, security etc. Used to encrypt the data/voice being transmitted on communication links such as telephone lines. Biometric systems using fingerprint and face recognition are now being extensively used for user authentication in banking applications as well as for access control in high security buildings.

Finance:

Financial dealing through cash and cheques are now slowly paving way for transactions using smart cards and ATM (Automatic Teller Machine, also expanded as Any Time Money) machines. Smart card, of the size of a credit card, has a small micro-controller and memory; and it interacts with the smart card reader! ATM machine and acts as an electronic wallet. Smart card technology has the capability of ushering in a cashless society. Well, the list goes on. It is no exaggeration to say that eyes wherever you go, you can see, or at least feel, the work of an embedded system.

Overview of Embedded System Architecture

Every embedded system consists of custom-built hardware built around a Central Processing Unit (CPU). This hardware also contains memory chips onto which the software is loaded. The software residing on the memory chip is also called the ‘firmware’. The embedded system architecture can be represented as a layered architecture as shown in Fig. The operating system runs above the hardware, and the application software runs above the operating system. The same architecture is applicable to any computer including a desktop computer. However, there are significant differences. It is not compulsory to have an operating system in every embedded system. For small appliances such as remote control units, air conditioners, toys etc., there is no need for an operating system and you can write only the software specific to that application. For applications involving complex processing, it is advisable to have an operating system. In such a case, you need to integrate the application software with the operating system and then transfer the entire software on to the memory chip. Once the software is transferred to the memory chip, the software will continue to run for a long time you don’t need to reload new software.

Now, let us see the details of the various building blocks of the hardware of an embedded system. As shown in Fig. the building blocks are;

· Central Processing Unit (CPU)

· Memory (Read-only Memory and Random Access Memory)

· Input Devices

· Output devices

· Communication interfaces

· Application-specific circuitry

Central Processing Unit (CPU):

The Central Processing Unit (processor, in short) can be any of the following: microcontroller, microprocessor or Digital Signal Processor (DSP). A micro-controller is a low-cost processor. Its main attraction is that on the chip itself, there will be many other components such as memory, serial communication interface, analog-to digital converter etc. So, for small applications, a micro-controller is the best choice as the number of external components required will be very less. On the other hand, microprocessors are more powerful, but you need to use many external components with them. D5P is used mainly for applications in which signal processing is involved such as audio and video processing.

Memory:

The memory is categorized as Random Access Memory (RAM) and Read Only Memory (ROM). The contents of the RAM will be erased if power is switched off to the chip, whereas ROM retains the contents even if the power is switched off. So, the firmware is stored in the ROM. When power is switched on, the processor reads the ROM; the program is program is executed.

Input Devices:

Unlike the desktops, the input devices to an embedded system have very limited capability. There will be no keyboard or a mouse, and hence interacting with the embedded system is no easy task. Many embedded systems will have a small keypad-you press one key to give a specific command. A keypad may be used to input only the digits. Many embedded systems used in process control do not have any input device for user interaction; they take inputs from sensors or transducers 1’fnd produce electrical signals that are in turn fed to other systems.

Output Devices:

The output devices of the embedded systems also have very limited capability. Some embedded systems will have a few Light Emitting Diodes (LEDs) to indicate the health status of the system modules, or for visual indication of alarms. A small Liquid Crystal Display (LCD) may also be used to display some important parameters.

Communication Interfaces:

The embedded systems may need to, interact with other embedded systems at they may have to transmit data to a desktop. To facilitate this, the embedded systems are provided with one or a few communication interfaces such as RS232, RS422, RS485, Universal Serial Bus (USB), and IEEE 1394, Ethernet etc.

Application-Specific Circuitry:

Sensors, transducers, special processing and control circuitry may be required fat an embedded system, depending on its application. This circuitry interacts with the processor to carry out the necessary work. The entire hardware has to be given power supply either through the 230 volts main supply or through a battery. The hardware has to design in such a way that the power consumption is minimized.

2.2 BLOCK DIAGRAM

Motor

Driver

Micro

Controller

Power Supply

Motor Driver

Robot mechanism

Temperature Sensor

Moisture

Sensor

GSM

LCD

Comparator

2.3 MODULES

1. POWER SUPPLY

The power supply section is the section which provide +5V for the components to work. IC LM7805 is used for providing a constant power of +5V.

The ac voltage, typically 220V, is connected to a transformer, which steps down that ac voltage down to the level of the desired dc output. A diode rectifier then provides a full-wave rectified voltage that is initially filtered by a simple capacitor filter to produce a dc voltage. This resulting dc voltage usually has some ripple or ac voltage variation.

A regulator circuit removes the ripples and also retains the same dc value even if the input dc voltage varies, or the load connected to the output dc voltage changes. This voltage regulation is usually obtained using one of the popular voltage regulator IC units.

Figure. Block Diagram of Power Supply

Transformer

Transformers convert AC electricity from one voltage to another with little loss of power. Transformers work only with AC and this is one of the reasons why mains electricity is AC.

Step-up transformers increase voltage, step-down transformers reduce voltage. Most power supplies use a step-down transformer to reduce the dangerously high mains voltage (230V in India) to a safer low voltage.

The input coil is called the primary and the output coil is called the secondary. There is no electrical connection between the two coils; instead they are linked by an alternating magnetic field created in the soft-iron core of the transformer. Transformers waste very little power so the power out is (almost) equal to the power in. Note that as voltage is stepped down current is stepped up. The transformer will step down the power supply voltage (0-230V) to (0- 6V) level. Then the secondary of the potential transformer will be connected to the bridge rectifier, which is constructed with the help of PN junction diodes. The advantages of using bridge rectifier are it will give peak voltage output as DC.

Rectifier

There are several ways of connecting diodes to make a rectifier to convert AC to DC. The bridge rectifier is the most important and it produces full-wave varying DC. A full-wave rectifier can also be made from just two diodes if a centre-tap transformer is used, but this method is rarely used now that diodes are cheaper. A single diode can be used as a rectifier but it only uses the positive (+) parts of the AC wave to produce half-wave varying DC

Bridge Rectifier

When four diodes are connected as shown in figure, the circuit is called as bridge rectifier. The input to the circuit is applied to the diagonally opposite corners of the network, and the output is taken from the remaining two corners. Let us assume that the transformer is working properly and there is a positive potential, at point A and a negative potential at point B. the positive potential at point A will forward bias D3 and reverse bias D4.

Figure.Bridge Rectifier

The negative potential at point B will forward bias D1 and reverse D2. At this time D3 and D1 are forward biased and will allow current flow to pass through them; D4 and D2 are reverse biased and will block current flow.

One advantage of a bridge rectifier over a conventional full-wave rectifier is that with a given transformer the bridge rectifier produces a voltage output that is nearly twice that of the conventional full-wave circuit.

i. The main advantage of this bridge circuit is that it does not require a special centre tapped transformer, thereby reducing its size and cost.

ii. The single secondary winding is connected to one side of the diode bridge network and the load to the other side as shown below.

iii. The result is still a pulsating direct current but with double the frequency.

Figure: Output Waveform of DC

Smoothing

Smoothing is performed by a large value electrolytic capacitor connected across the DC supply to act as a reservoir, supplying current to the output when the varying DC voltage from the rectifier is falling. The capacitor charges quickly near the peak of the varying DC, and then discharges as it supplies current to the output.

Voltage Regulators

Voltage regulators comprise a class of widely used ICs. Regulator IC units contain the circuitry for reference source, comparator amplifier, control device, and overload protection all in a single IC. IC units provide regulation of either a fixed positive voltage, a fixed negative voltage, or an adjustably set voltage. The regulators can be selected for operation with load currents from hundreds of milli amperes to tens of amperes, corresponding to power ratings from milli watts to Tens of watts.

A fixed three-terminal voltage regulator has an unregulated dc input voltage, Vi, applied to one input terminal, a regulated dc output voltage, Vo, from a second terminal, with the third terminal connected to ground.

The series 78 regulators provide fixed positive regulated voltages from 5 to 24 volts. Similarly, the series 79 regulators provide fixed negative regulated voltages from 5 to 24 volts. Voltage regulator ICs are available with fixed (typically 5, 12 and 15V) or variable output voltages. They are also rated by the maximum current they can pass. Negative voltage regulators are available, mainly for use in dual supplies. Most regulators include some automatic protection from excessive current ('overload protection') and overheating ('thermal protection').

Many of the fixed voltage regulator ICs has 3 leads and look like power transistors, such as the 7805 +5V 1Amp regulator. They include a hole for attaching a heat sink if necessary.

Figure. Regulator

Figure. Circuit Diagram of Power Supply

2.4. MICROCONTROLLER

A Microcontroller (or MCU) is a computer-on-a-chip used to control electronic devices. It is a type of microprocessor emphasizing self-sufficiency and cost-effectiveness, in contrast to a general-purpose microprocessor (the kind used in a PC). A typical microcontroller contains all the memory and interfaces needed for a simple application, whereas a general purpose microprocessor requires additional chips to provide these functions.

A microcontroller is a single integrated circuit with the following key features:

- central processing unit - ranging from small and simple 8-bit processors to sophisticated 32- or 64-bit processors

- input/output interfaces such as serial ports

- RAM for data storage

- ROM, EEPROM or Flash memory for program storage

- clock generator - often an oscillator for a quartz timing crystal, resonator or RC circuit

Microcontrollers are inside many kinds of electronic equipment (see embedded system). They are the vast majority of all processor chips sold. Over 50% are "simple" controllers, and another 20% are more specialized digital signal processors (DSPs) (ref?). A typical home in a developed country is likely to have only one or two general-purpose microprocessors but somewhere between one and two dozen microcontrollers. A typical mid range vehicle has as many as 50 or more microcontrollers. They can also be found in almost any electrical device: washing machines, microwave ovens, telephones etc.

FEATURES

• Compatible with MCU®-51 Products

• 8K Bytes of In-System Programmable (ISP) Flash Memory

– Endurance: 10,000 Write/Erase Cycles

• 4.0V to 5.5V Operating Range

• Fully Static Operation: 0 Hz to 33 MHz

• 256 x 8-bit Internal RAM

• 32 Programmable I/O Lines

• Three 16-bit Timer/Counters

• Eight Interrupt Sources

• Full Duplex UART Serial Channel

• Low-power Idle and Power-down Modes

• Interrupt Recovery from Power-down Mode

• Fast Programming Time

• Flexible ISP Programming (Byte and Page Mode)

Description

The AT89S52 is a low-power, high-performance CMOS 8-bit microcontroller with 8K bytes of in-system programmable Flash memory. The device is manufactured using Atmel’s high-density nonvolatile memory technology and is compatible with the industry-standard 80C51 instruction set and pin-out. The on-chip Flash allows the program memory to be reprogrammed in-system or by a conventional nonvolatile memory programmer. By combining a versatile 8-bit CPU with in-system programmable Flash on a monolithic chip, the Atmel AT89S52 is a powerful microcontroller which provides a highly-flexible and cost-effective solution to many embedded control applications.

The AT89S52 provides the following standard features: 8K bytes of Flash, 256 bytes of RAM, 32 I/O lines, three 16-bit timer/counters, a six-vector two-level interrupt architecture, a full duplex serial port, on-chip oscillator, and clock circuitry. In addition, the AT89S52 is designed with static logic for operation down to zero frequency and supports two software selectable power saving modes. The Idle Mode stops the CPU while allowing the RAM, timer/counters, serial port, and interrupt system to continue functioning. The Power-down mode saves the RAM con-tents but freezes the oscillator, disabling all other chip functions until the next interrupt or hardware reset.

Fig. BLOCK DIAGRAM OF 8052

PIN DIAGRAM

Fig. PIN DIAGRAM OF 8052

PIN DESCRIPTION

40 pin VCC Supply voltage.

20 pin GND Ground.

Port 0

Port 0 is an 8-bit open drain bidirectional I/O port. As an output port, each pin can sink eight TTL inputs. When 1s are written to port 0 pins, the pins can be used as high-impedance inputs. Port 0 can also be configured to be the multiplexed low-order address/data bus during accesses to external program and data memory. Port 0 also receives the code bytes during Flash programming and outputs the code bytes during program verification. External pull-ups are required during program verification.

Port 1

Port 1 is an 8-bit bidirectional I/O port with internal pull-ups. The Port 1 output buffers can sink/source four TTL inputs. When 1s are written to Port 1 pins, they are pulled high by the internal pull-ups and can be used as inputs. As inputs, Port 1 pins that are externally being pulled low will source current (IIL) because of the internal pull-ups.

Port 2

Port 2 is an 8-bit bidirectional I/O port with internal pull-ups. The Port 2 output buffers can sink/source four TTL inputs. When 1s are written to Port 2 pins, they are pulled high by the internal pull-ups and can be used as inputs. As inputs, Port 2 pins that are externally being pulled low will source current because of the internal pull-ups. In this application, Port 2 uses strong internal pull-ups when emitting 1s. During accesses to external data memory that use 8-bit addresses, Port 2 emits the contents of the P2 Special Function Register. Port 2 also receives the high-order address bits and some control signals during Flash programming and verification.

Port 3

Port 3 is an 8-bit bidirectional I/O port with internal pull-ups. The Port 3 output buffers can sink/source four TTL inputs. When 1s are written to Port 3 pins, they are pulled high by the internal pull-ups and can be used as inputs. As inputs, Port 3 pins that are externally being pulled low will source current because of the pull-ups. Port 3 also serves the functions of various special features of the AT89S52, as shown in the following table.

RST

Reset input. A high on this pin for two machine cycles while the oscillator is running resets the device.

WR

It is active low write O/P control signal. During External RAM (Data memory). Write to external RAM.

RD

It is active low read O/P control signal. During External RAM (Data memory). Read from External RAM.

XTAL1 Input to the inverting oscillator amplifier and input to the internal clock operating circuit.

XTAL2 Output from the inverting oscillator amplifier.

ALE/PROG

Address Latch Enable (ALE) is an output pulse for latching the low byte of the address during accesses to external memory. This pin is also the program pulse input (PROG) during Flash programming. In normal operation, ALE is emitted at a constant rate of 1/6 the oscillator frequency and may be used for external timing or clocking purposes. If desired, ALE operation can be disabled by setting bit 0 of SFR location 8EH. With the bit set, ALE is active only during a MOVX or MOVC instruction. Otherwise, the pin is weakly pulled high. Setting the ALE-disable bit has no effect if the microcontroller is in external execution mode.

PSEN

Program Store Enable (PSEN) is the read strobe to external program memory. When the AT89S52 is executing code from external program memory, PSEN is activated twice each machine cycle.

EA

External Access Enable. EA must be strapped to GND in order to enable the device to fetch code from external program memory locations starting at 0000H up to FFFFH. Note, however, that if lock bit 1 is programmed, EA will be internally latched on reset. EA should be strapped to VCC for internal program executions.

ARCHITECTURE OF 8052

Fig. INTERNAL ARCHITECTURE OF 8052

REGISTERS

Registers and accumulators serve as temporary memory locations during CPU operations. The exchange of information among them takes place through one or more internal buses.  The length of each register is equal to the width of the internal data bus.

Registers are categorized as general-purpose registers and special-purpose registers.

A general-purpose register may be used as an accumulator or as a data register for (arithmetic) and logic operations. An accumulator is a register for storing the results of an arithmetic operation. A general-purpose register may also be used as an address register.

A special-purpose register is dedicated to a specific function. Some of these registers are the program counter (PC), the instruction register (IR), the address register (AR), the status register, and the stack pointer (SP).

The program counter holds the address of the next instruction to be executed. When the CPU executes a branch instruction telling it to branch to another part of the program, the new address is loaded into the program counter and the sequential order resumes.

The instruction register extracts the operation code (op code) from an instruction. An instruction consists of op code and operand(s). The contents of the instruction register are decoded by the control unit. The status register or condition code register consists of status or flag bits and control bits.

The flag bits are: carry/borrow (C), overflow (V), negative (N), zero (Z), and half-carry (H). The logic states of the flag bits are used by the CPU’s branch instructions to make decisions. The program counter sends its contents (address) to the address register. The address register then sends this address to the address bus.

A stack is a specially reserved area in memory where information is stored or removed in a last-in-first-out (LIFO) fashion. The stack pointer points to the next free location on the stack. That is, it holds the address of the top of the stack. Each time data is stored in the stack, the stack pointer is automatically decremented and each time data is retrieved from the stack, the stack pointer is automatically incremented.

Memory Organization

MCU-51 devices have a separate address space for Program and Data Memory. Up to 64K bytes each of external Program and Data Memory can be addressed.

The instructions to be executed by the microcontroller CPU and the data to be operated on by these instructions are stored in memory. When the CPU accesses the information stored in memory, it is performing a read operation. When the CPU sends information to be stored in the memory, it is performing a write operation. Memory is classified as internal and external memory. Internal memory is on-chip memory and is a semiconductor type with low capacity and high speeds. External memory is outside the chip and includes the semiconductor type and serial memory such as magnetic disks, magnetic tapes, and bubble memory. Semiconductor memory may be volatile or nonvolatile. Volatile memory loses its contents after the power is removed from the memory chip. Nonvolatile memory does not lose its contents when power is removed. The nonvolatile memory can store information permanently or at least semi-permanently (ten years or more). Volatile memory includes RAM while nonvolatile memory includes ROM, EPROM, EEPROM, and battery-backed RAM.

RAM (random access memory) comprises DRAM (Dynamic RAM) and SRAM (Static RAM). Each storage cell of DRAM consists of a capacitor and a MOSFET (Metal Oxide Semi conductor Field Effect Transistor). If the capacitor is charged, a logic 1 is stored in the cell. If the capacitor has no charge, a logic 0 is stored in the cell. The charge stored in the capacitors dissipates fast because of leakage. Therefore, a DRAM has to be periodically refreshed otherwise its contents will fade away after some time even in the presence of the power to memory chip.

The storage cell of a ROM (Read-only memory) consists of a BJT (Bipolar Junction Transistor). A ROM may be programmed at the manufacturing stage using a mask process. Such a ROM is called mask-programmed ROM. The contents of a mask-programmed ROM cannot be changed once it is programmed.

Data Memory

The AT89S52 implements 256 bytes of on-chip RAM. The upper 128 bytes occupy a parallel address space to the Special Function Registers. This means that the upper 128 bytes have the same addresses as the SFR space but are physically separate from SFR space.

When an instruction accesses an internal location above address 7FH, the address mode used in the instruction specifies whether the CPU accesses the upper 128 bytes of RAM or the SFR space. Instructions which use direct addressing access the SFR space.

### UART (Universal Asynchronous Receiver and Transmitter)

One of the microcontroller features making it so powerful is an integrated UART, better known as a serial port. It is a full-duplex port, thus being able to transmit and receive data simultaneously and at different baud rates. Without it, serial data send and receive would be an enormously complicated part of the program in which the pin state is constantly changed and checked at regular intervals. When using UART, all the programmer has to do is to simply select serial port mode and baud rate. When it's done, serial data transmit is nothing but writing to the SBUF register, while data receive represents reading the same register. The microcontroller takes care of not making any error during data transmission.

TIMERS

Timer is use to generate time delay, a timer always counts up. It doesn’t matter whether the timer is being used as a timer, a counter, or a baud rate generator: A timer is always incremented by the microcontroller. There are 3 timers i.e., Timer 0, Timer 1 and Timer 2.

Timer 0 and 1

Timer 0 and Timer 1 in the AT89S52 operate the same way as Timer 0 and Timer 1 in the 8051.

Timer T0 is a 16 bit timer. The 16-bit  Timer 0 is accessed as low byte and high byte. the low byte is called TL0(Timer 0 low byte)and the high byte is referred to as TH0(Timer 0 high byte).

As like Timer 1 is also 16-bit timer is split into two bytes, referred to as TL1 (Timer 1 low byte) and TH1(Timer 1 high byte). These timer are accessible in the same way  as the Timer 0.

Timer 2

Timer 2 is a 16-bit Timer/Counter that can operate as either a timer or an event counter. The type of operation is selected by bit C/T2 in the SFR T2CON (shown in Table 5-2). Timer 2 has three operating modes: capture, auto-reload (up or down counting), and baud rate generator. Timer 2 consists of two 8-bit registers, TH2 and TL2. In the Timer function, the TL2 register is incremented every machine cycle. Since a machine cycle consists of 12 oscillator periods, the count rate is 1/12 of the oscillator frequency.

In the Counter function, the register is incremented in response to a 1-to-0 transition at its corresponding external input pin, T2. In this function, the external input is sampled during S5P2 of every machine cycle. When the samples show a high in one cycle and a low in the next cycle, the count is incremented. The new count value appears in the register during S3P1 of the cycle following the one in which the transition was detected. Since two machine cycles (24 oscillator periods) are required to recognize a 1-to-0 transition, the maximum count rate is 1/24 of the oscillator frequency. To ensure that a given level is sampled at least once before it changes, the level should be held for at least one full machine cycle.

INTERRUPTS

The AT89S52 has a total of six interrupt vectors: two external interrupts (INT0 and INT1), three timer interrupts (Timers 0, 1, and 2), and the serial port interrupt.

Each of these interrupt sources can be individually enabled or disabled by setting or clearing a bit in Special Function Register IE. IE also contains a global disable bit, EA, which disables all interrupts at once.

Timer 2 interrupt is generated by the logical OR of bits TF2 and EXF2 in register T2CON. Neither of these flags is cleared by hardware when the service routine is vectored to. In fact, the service routine may have to determine whether it was TF2 or EXF2 that generated the interrupt, and that bit will have to be cleared in software.

The Timer 0 and Timer 1 flags, TF0 and TF1, uses the cycle in which the timers overflow. The values are then polled by the circuitry in the next cycle. However, the Timer 2 flag, TF2, is set at S2P2 and is polled in the same cycle in which the timer overflows.

2.5 LIQUID CRYSTAL DISPLAY

2.5.1 INTRODUCTION

LCD (Liquid Crystal Display) screen is an electronic display module and find a wide range of applications. A 16x2 LCD display is very basic module and is very commonly used in various devices and circuits. These modules are preferred over seven segments and other multi segment LEDs. The reasons being: LCDs are economical; easily programmable; have no limitation of displaying special & even custom characters (unlike in seven segments), animations and so on.

A 16x2 LCD means it can display 16 characters per line and there are 2 such lines. In this LCD each character is displayed in 5x7 pixel matrix. This LCD has two registers, namely, Command and Data.

The command register stores the command instructions given to the LCD. A command is an instruction given to LCD to do a predefined task like initializing it, clearing its screen, setting the cursor position, controlling display etc. The data register stores the data to be displayed on the LCD. The data is the ASCII value of the character to be displayed on the LCD.

Fig: 16x2 LCD

Introduction

The most commonly used Character based LCDs are based on Hitachi's HD44780 controller or other which are compatible with HD44580.

2.5.2 Pin Description

Most LCDs with 1 controller has 14 Pins and LCDs with 2 controller has 16 Pins (two pins are extra in both for back-light LED connections). Pin description is shown in the table below.

### Pin Configuration table for a 16X2 LCD character display:-

| Pin Number | Symbol | Function |
| --- | --- | --- |
| 1 | Vss | Ground Terminal |
| 2 | Vcc | Positive Supply |
| 3 | Vdd | Contrast adjustment |
| 4 | RS | Register Select; 0→Instruction Register, 1→Data Register |
| 5 | R/W | Read/write Signal; 1→Read, 0→ Write |
| 6 | E | Enable; Falling edge |
| 7 | DB0 | Bi-directional data bus, data transfer is performed once, thru DB0 to DB7, in the case of interface data length is 8-bits; and twice, through DB4 to DB7 in the case of interface data length is 4-bits. Upper four bits first then lower four bits. |
| 8 | DB1 |  |
| 9 | DB2 |  |
| 10 | DB3 |  |
| 11 | DB4 |  |
| 12 | DB5 |  |
| 13 | DB6 |  |
| 14 | DB7 |  |
| 15 | LED-(K) | Back light LED cathode terminal |
| 16 | LED+(A) | Back Light LED anode terminal |

Fig: Pin Configuration of LCD

### 2.5.3 Data/Signals/Execution of LCD

LCD accepts two types of signals, one is data, and another is control. These signals are recognized by the LCD module from status of the RS pin. Now data can be read also from the LCD display, by pulling the R/W pin high. As soon as the E pin is pulsed, LCD display reads data at the falling edge of the pulse and executes it, same for the case of transmission.

LCD display takes a time of 39-43µS to place a character or execute a command. Except for clearing display and to seek cursor to home position it takes 1.53ms to 1.64ms. Any attempt to send any data before this interval may lead to failure to read data or execution of the current data in some devices. Some devices compensate the speed by storing the incoming data to some temporary registers.

Instruction Register (IR) and Data Register (DR)

There are two 8-bit registers in HD44780 controller Instruction and Data register. Instruction register corresponds to the register where you send commands to LCD e.g LCD shift command, LCD clear, LCD address etc. and Data register is used for storing data which is to be displayed on LCD. when send the enable signal of the LCD is asserted, the data on the pins is latched in to the data register and data is then moved automatically to the DDRAM and hence is displayed on the LCD.  Data Register is not only used for sending data to DDRAM but also for CGRAM, the address where you want to send the data, is decided by the instruction you send to LCD. We will discuss more on LCD instruction set further in this tutorial.

2.5.4 Commands and Instruction set

Only the instruction register (IR) and the data register (DR) of the LCD can be controlled by the MCU. Before starting the internal operation of the LCD, control information is temporarily stored into these registers to allow interfacing with various MCUs, which operate at different speeds, or various peripheral control devices. The internal operation of the LCD is determined by signals sent from the MCU. These signals, which include register selection signal (RS), read/write signal (R/W), and the data bus (DB0 to DB7), make up the LCD instructions (Table 3). There are four categories of instructions that:

- Designate LCD functions, such as display format, data length, etc.

- Set internal RAM addresses

- Perform data transfer with internal RAM

- Perform miscellaneous functions

Fig: Code for LCD

Although looking at the table you can make your own commands and test them. Below is a brief list of useful commands which are used frequently while working on the LCD.

List of Command

| No. | Instruction | Hex | Decimal |
| --- | --- | --- | --- |
| 1 | Function Set: 8-bit, 1 Line, 5x7 Dots | 0x30 | 48 |
| 2 | Function Set: 8-bit, 2 Line, 5x7 Dots | 0x38 | 56 |
| 3 | Function Set: 4-bit, 1 Line, 5x7 Dots | 0x20 | 32 |
| 4 | Function Set: 4-bit, 2 Line, 5x7 Dots | 0x28 | 40 |
| 5 | Entry Mode | 0x06 | 6 |
| 6 | Display off Cursor off (clearing display without clearing DDRAM content) | 0x08 | 8 |
| 7 | Display on Cursor on | 0x0E | 14 |
| 8 | Display on Cursor off | 0x0C | 12 |
| 9 | Display on Cursor blinking | 0x0F | 15 |
| 10 | Shift entire display left | 0x18 | 24 |
| 12 | Shift entire display right | 0x1C | 30 |
| 13 | Move cursor left by one character | 0x10 | 16 |
| 14 | Move cursor right by one character | 0x14 | 20 |
| 15 | Clear Display (also clear DDRAM content) | 0x01 | 1 |
| 16 | Set DDRAM address or cursor position on display | 0x80+add* | 128+add* |
| 17 | Set CGRAM address or set pointer to CGRAM location | 0x40+add** | 64+add** |

Table: Frequently used commands and instructions for LCD

* DDRAM address given in LCD basics section see Figure 2,3,4
** CGRAM address from 0x00 to 0x3F, 0x00 to 0x07 for char1 and so on.

2.5.5 Liquid crystal displays interfacing with Controller

The LCD standard requires 3 control lines and 8 I/O lines for the data bus.

• 8 data pins D7:D0

Bi-directional data/command pins.
Alphanumeric characters are sent in ASCII format.

• RS:  Register Select

RS = 0 -> Command Register is selected
RS = 1 -> Data Register is selected

• R/W: Read or Write

0 -> Write, 1 -> Read

• E: Enable (Latch data)

Used to latch the data present on the data pins.
A high-to-low edge is needed to latch the data.

2.6 Temperature sensor

Temperature is the most-measured process variable in industrial automation. Most commonly, a temperature sensor is used to convert temperature value to an electrical value. Temperature Sensors are the key to read temperatures correctly and to control temperature in industrials applications.

A large distinction can be made between temperature sensor types. Sensors differ a lot in properties such as contact-way, temperature range, calibrating method and sensing element. The temperature sensors contain a sensing element enclosed in housings of plastic or metal. With the help of conditioning circuits, the sensor will reflect the change of environmental temperature.

Temperature sensor (LM35)

## Pin Definition

The definition of gray-scale sensor pin is

- Signal Output

- GND

- Power

Features of Temperature Sensor

- Calibrated directly in Celsius (centigrade)

- 0.50 C Ensured accuracy (at +250 C)

- Suitable for remote applications

- Operate from 4 to 30 V

- Low cost due to wafer-level trimming

2.6.1 Specifications

- Type: Analog

- Sensitivity: 10mV per degree Celsius

- Functional range: 0 degree Celsius to 100 degree Celsius

Description

The LM35 series are precision integrated-circuit temperature sensors, with an output voltage linearly proportional to the Centigrade temperature. Thus LM35 has an advantage over linear temperature sensors calibrated in ° Kelvin, as the user is not required to subtract a large constant voltage from the output to obtain convenient Centigrade scaling.

In the temperature functional module we developed, we use the LM34 series of temperature sensors. The LM34 series are precision integrated-circuit temperature sensors, whose output voltage is linearly proportional to the Fahrenheit temperature. The LM34 thus has an advantage over linear temperature sensors calibrated in degrees Kelvin, as the user is not required to subtract a large constant voltage from its output to obtain convenient Fahrenheit scaling. The LM34 does not require any external calibration or trimming to provide typical accuracies of ±1.2°F at room temperature and ±11.2°F over a full -50 to +300°F temperature range. The LM34 is rated to operate over a -50° to +300°F temperature range.

Figure 1 Circuit diagram for the LM34 temperature sensor functional module

It is easy to include the LM34 series in a temperature measuring application. The output voltage of LM34 is linearly proportional to the Fahrenheit temperature, it has a Linear +10.0 mV/°F scale factor which means that you will get  n*10.0 mV output voltage if the environment  temperature is n°F.

The LM34 series is available packaged in hermetic TO-46 transistor packages, while the LM34C, LM34CA and LM34D are also available in the plastic TO-92 transistor package. The LM34D is also available in an 8-lead surface mount small outline package. In our functional module, LM34H in metal can package (TO-46) is used in the functional module, it is very important to know that the wiring of sensor should be based on the positions of the leading pins in different packages.

Figure 2 Package Diagram of LM34

# 2.	dESCRIPTION OF TEMPERATURE SENSOR FUNCTIONAL MODULE

The temperature sensor functional module consists of two parts: the function module box and the probe head. The LM34 temperature sensor is mounted on the probe head. Be careful to make sure that the sensor is properly mounted on the probe head. (refer to Figure 4 Labeled picture of the probe head.)

GND

GND

+5V

OUTPUT VOLTAGE

PROBE HEAD

SENSOR

Figure 3	Labeled picture of the temperature sensor circuit functional module.

By replacing the LM34 with another precision integrated-circuit temperature sensor LM35, we can easily get an output voltage proportional to the centigrade temperature. The LM35 sensor has a linear +10.0 mV/°C scale factor and a temperature range from  -55°C to +150°C.In fact LM34 and LM35 are among the same series of temperature sensors so that they can be easily exchanged in different applications. The wiring for LM 35 is the same as that of LM34. Please refer to the datasheets of LM34 and LM35 for more detailed packaging and features information.

2.6.2 APPLICATIONS

The LM35 is applied easily in the same way as other integrated-circuit temperature sensors. Glue or cement the device to a surface and the temperature should be within about 0.01°C of the surface temperature.

This presumes that the ambient air temperature is almost the same as the surface temperature. If the air temperature were much higher or lower than the surface temperature, the actual temperature of the LM35 die would be at an intermediate temperature between the surface temperature and the air temperature, which is especially true for the TO-92 plastic package where the copper leads are the principal thermal path to carry heat into the device, so its temperature might be closer to the air temperature than to the surface temperature.

To minimize this problem, ensure that the wiring to the LM35, as it leaves the device, is held at the same temperature as the surface of interest. The easiest way to do this is to cover up these wires with a bead of epoxy which will insure that the leads and wires are all at the same temperature as the surface, and that the temperature of the LM35 die is not affected by the air temperature.

2.7 INTERFACING CIRCUIT (LM358)

A Comparator is a device that compares two voltages or currents and outputs a digital signal indicating which is larger. It has two analog input terminals  and  and one binary digital output . The output is ideally

Features

- Wide Range of Supply Voltages: 3 V to 32 V (LM2904: 3 V to 26 V) or Dual Supplies

- Common-Mode Range Extends to Negative Supply

- Low Input Bias and Offset Current

- Differential Input Voltage Range Equal to Maximum-Rated Supply Voltage :±32 V (±26V for LM2904)

- Single And Split Supply Operation

- Internal Frequency Compensation

- Package: SOP-8, DIP

Description

The LM358 consist of two independent, high gain, internally frequency compensated operational amplifiers which were designed specifically to operate from a single power supply over a wide range of voltage. Operation from split power supplies is also possible and the low power supply current drain is independent of the magnitude of the power supply voltage. Application areas include transducer amplifier, DC gain blocks and all the conventional OP-AMP circuits which now can be easily implemented in single power supply systems.

Internal Block Diagram

Application

Application areas include transducer amplifiers, dc gain blocks and all the conventional op amp circuits which now can be more easily implemented in single power supply systems. For example, the LM158 series can be directly operated off of the standard +5V power supply voltage which is used in digital systems and will easily provide the required interface electronics without requiring the additional ±15V power supplies.

2.8 HUMIDITY SENSOR

Humidity sensor is an instrument used for measuring the moisture content in the environment. Humidity measurement instruments usually rely on measurements of some other quantity such as temperature, pressure, mass or a mechanical or electrical change in a substance as moisture is absorbed. By calibration and calculation, these measured quantities can lead to a measurement of humidity.

### 2.8.1 Features

- Relative humidity sensor
- Two point calibrated with capacitor type sensor, excellent performance
- Frequency output type, can be easily integrated with user application system
- Very low power consumption
- No extra components needed

Performance and Reliability

Each sensor is designed to provide enhanced sensitivity, response time, stability and reliability. Although humidity products are typically standardized and platform-based, they deliver superior performance right out of the box.

Sensor construction consists of a planar capacitor with a second polymer layer to protect against dirt, dust, oils, and other hazards. Laser trimmed for stable, low drift performance. Printed circuit board or surface mount terminations provide application flexibility.

RELEVANT MOISTURE TERMS

To mention moisture levels, variety of terminologies are used. The study of water vapour concentration in air as a function of temperature and pressure falls under the area of psychometrics. Psychometrics deals with the thermodynamic properties of moist gases while the term “humidity’ simply refers to the presence of water vapour in air or other carrier gas.

Humidity measurement determines the amount of water vapor present in a gas that can be a mixture, such as air, or a pure gas, such as nitrogen or argon. Various terms used to indicate moisture levels are tabulated in the table below:

| S.No | Term | Definition | Unit |
| --- | --- | --- | --- |
| 1 | Absolute Humidity (Vapor Concentration) | Ratio of mass(vapour) to volume. | grams/m3 |
| 2 | Mixing Ratio OR Mass Ratio | Ratio of mass(vapour) to mass(dry gas) | grams/m3 |
| 3 | Relative Humidity | Ratio of mass(vapour) to mass(saturated vapour) OR ratio of actual vapor pressure to saturation vapor pressure. | % |
| 4 | Specific Humidity | Ratio of mass(vapour) to total mass. | % |
| 5 | Dew Point | Temperature(above 0°C) at which the water vapor in a gas condenses to liquid water) | °C |
| 6 | Frost Point | Temperature(below 0°C) at which the water vapor in a gas condenses to ice |  |
| 7 | Volume Ratio | Ratio of partial pressure(vapour) to partial pressure (dry gas) | % by volume |
| 8 | PPM by Volume | Ratio of volume(vapour) X 106 to volume(dry gas) | PPMV |
| 9 | PPM by Weight | PPMV X | PPMW |

Most commonly used units for humidity measurement are Relative Humidity (RH), Dew/Frost point (D/F PT) and Parts Per Million (PPM). RH is a function of temperature, and thus it is a relative measurement. Dew/Frost point is a function of the pressure of the gas but is independent of temperature and is therefore defined as absolute humidity measurement. PPM is also an absolute measurement.

Dew points and frost points are often used when the dryness of the gas is important. Dew point is also used as an indicator of water vapor in high temperature processes, such as industrial drying.

Mixing ratios, volume percent, and specific humidity are usually used when water vapor is either an impurity or a defined component of a process gas mixture used in manufacturing.

Correlation among RH, Dew/Frost point and PPMv is shown below:

HUMIDITY SENSING – CLASSIFICATION & PRINCIPLES

According to the measurement units, humidity sensors are divided into two types: Relative humidity(RH)sensors and absolute humidity(moisture) sensors. Most humidity sensors are relative humidity sensors and use different sensing principles.

A table showing important parameters of different types of humidity sensors is given below

| Active Material | Thermo-set Polymer | Thermoplastic Polymer | Thermoplastic Polymer | Bulk Thermoplastic | Bulk AlO3 | Lithium Chloride Film |
| --- | --- | --- | --- | --- | --- | --- |
| Substrate | Ceramic or Silicon | Ceramic or silicon | Polyester or mylar film | N/A | N/A | Ceramic |
| Sensed Parameter | Capacitance | Capacitance | Capacitance | Resistance | Resistance | Conductivity |
| Measured Parameter | %RH | %RH | %RH | %RH | %RH | %RH |
| RH Change | 0% to 100% | 0% to 100% | 0% to 100% | 20% to 100% | 2% to 90% | 15% to <100% |
| RH Accuracy | ±1% to ±5% | ±3% to ±5% | ±3% to ±5% | ±3% to ±10% | ±1% to ±5% | ±5% |
| Interchangability | ±2% to ±10% RH | ±3% to ±20% RH | ±3% to ±20% RH | ±5% to ±25% RH | poor | ±3% to ±10% RH |
| Hysterisis | <1% to 3% RH | 2% to 5% RH | 2% to 5% RH | 3% to 6% RH | <2% RH | very poor |
| Linearity | ±1% RH | ±1% RH | ±2% RH | Poor | poor | Very poor |
| Risetime | 15 s to 60 s | 15 s to 90 s | 15 s to 90 s | 2 min to 5 min | 3 min to 5 min | 3 min to 5 min |
| Temperature Range | -40 °C to 185 °C | -30 °C to 190 °C | -25°C to 100 °C | 10 °C to 40 °C | -10 °C to 75 °C | - |
| Long Term Stability | ±1%RH/5 yr | ±1%RH/yr | ±1%RH/yr | ±3%RH/yr | ±3% RH/yr | >1% RH/°C |

Sensing Principle

Humidity measurement can be done using dry and wet bulb hygrometers, dew point hygrometers, and electronic hygrometers. There has been a surge in the demand of electronic hygrometers, often called humidity sensors.

Electronic type hygrometers or humidity sensors can be broadly divided into two categories: one employs capacitive sensing principle, while other use resistive effects.

2.8.2 Applications

Besides greenhouses and industrial spaces, hygrometers are also used in some incubators (egg), saunas, humidors and museums. They are also used in the care of wooden musical instruments such as guitars and violins which can be damaged by improper humidity conditions. In residential settings, hygrometers are used to aid humidity control (too low humidity damages human skin and body, while too high humidity favors growth of mildew and dust mite). Hygrometers are also used in the coating industry because the application of paint and other coatings may be very sensitive to humidity and dew point.

2.9 DRIVER CIRCUIT (L293D)

INTRODUCTION

L293D IC generally comes as a standard 16-pin DIP (dual-in line package). This motor driver IC can simultaneously control two small motors in either direction; forward and reverse with just 4 microcontroller pins (if you do not use enable pins).

Some of the features (and drawbacks) of this IC are:

- Output current capability is limited to 600mA per channel with peak output current limited to 1.2A (non-repetitive). This means you cannot drive bigger motors with this IC. However, most small motors used in hobby robotics should work. If you are unsure whether the IC can handle a particular motor, connect the IC to its circuit and run the motor with your finger on the IC. If it gets really hot, then beware... Also note the words "non-repetitive"; if the current output repeatedly reaches 1.2A, it might destroy the drive transistors.

- Supply voltage can be as large as 36 Volts. This means you do not have to worry much about voltage regulation.

- L293D has an enable facility which helps you enable the IC output pins. If an enable pin is set to logic high, then state of the inputs match the state of the outputs. If you pull this low, then the outputs will be turned off regardless of the input states

- The datasheet also mentions an "over temperature protection" built into the IC. This means an internal sensor senses its internal temperature and stops driving the motors if the temperature crosses a set point

- Another major feature of L293D is its internal clamp diodes. This flyback diode helps protect the driver IC from voltage spikes that occur when the motor coil is turned on and off (mostly when turned off)

- The logical low in the IC is set to 1.5V. This means the pin is set high only if the voltage across the pin crosses 1.5V which makes it suitable for use in high frequency applications like switching applications (upto 5KHz)

- Lastly, this integrated circuit not only drives DC motors, but can also be used to drive relay solenoids, stepper motors etc.

## Description

It works on the concept of H-bridge. H-bridge is a circuit which allows the voltage to be flown in either direction. As you know voltage need to change its direction for being able to rotate the motor in clockwise or anticlockwise direction, Hence H-bridge IC are ideal for driving a DC motor.

In a single l293d chip there two h-Bridge circuit inside the IC which can rotate two dc motor independently. Due its size it is very much used in robotic application for controlling DC motors. Given below is the pin diagram of a L293D motor controller.

There are two Enable pins on l293d. Pin 1 and pin 9, for being able to drive the motor, the pin 1 and 9 need to be high. For driving the motor with left H-bridge you need to enable pin 1 to high. And for right H-Bridge you need to make the pin 9 to high. If anyone of the either pin1 or pin9 goes low then the motor in the corresponding section will suspend working. It’s like a switch.

## Pin Diagram

## Fig showing pin diagram of L293D

Pin Description

| Pin No | Function | Name |
| --- | --- | --- |
| 1 | Enable pin for Motor 1; active high | Enable 1,2 |
| 2 | Input 1 for Motor 1 | Input 1 |
| 3 | Output 1 for Motor 1 | Output 1 |
| 4 | Ground (0V) | Ground |
| 5 | Ground (0V) | Ground |
| 6 | Output 2 for Motor 1 | Output 2 |
| 7 | Input 2 for Motor 1 | Input 2 |
| 8 | Supply voltage for Motors; 9-12V (up to 36V) | Vcc 2 |
| 9 | Enable pin for Motor 2; active high | Enable 3,4 |
| 10 | Input 1 for Motor 1 | Input 3 |
| 11 | Output 1 for Motor 1 | Output 3 |
| 12 | Ground (0V) | Ground |
| 13 | Ground (0V) | Ground |
| 14 | Output 2 for Motor 1 | Output 4 |
| 15 | Input2 for Motor 1 | Input 4 |
| 16 | Supply voltage; 5V (up to 36V) | Vcc 1 |

## Table Pin Description of L293D

## Working of L293D

The 4 input pins for this l293d, pin 2,7 on the left and pin 15 ,10 on the right as shown on the pin diagram. Left input pins will regulate the rotation of motor connected across left side and right input for motor on the right hand side. The motors are rotated on the basis of the inputs provided across the input pins as LOGIC 0 or LOGIC 1.

In simple you need to provide Logic 0 or 1 across the input pins for rotating the motor.

## Voltage Specification

VCC is the voltage that it needs for its own internal operation 5v; L293D will not use this voltage for driving the motor. For driving the motor it has a separate provision to provide motor supply VSS (V supply).  L293d will use this to drive the motor. It means if you want to operate a motor at 9V then you need to provide a Supply of 9V across VSS Motor supply.

The maximum voltage for VSS motor supply is 36V. It can supply a max current of 600mA per channel. Since it can drive motors Up to 36v hence you can drive pretty big motors with this l293d.

VCC pin 16 is the voltage for its own internal Operation. The maximum voltage ranges from 5v and up to 36v.

2.10 DC MOTOR

A DC motor  in simple words is a device that converts direct current(electrical energy) into mechanical energy. It’s of vital importance for the industry today.

A DC motor is designed to run on DC electric power. Two examples of pure DC designs are Michael Faraday's homo-polar motor (which is uncommon), and the ball bearing motor, which is (so far) a novelty.

By far the most common DC motor types are the brushed and brushless types, which use internal and external commutation respectively to create an oscillating AC current from the DC source—so they are not purely DC machines in a strict sense.

We in our project are using brushed DC Motor, which will operate in the ratings of 12v DC 0.6A.

The speed of a DC motor can be controlled by changing the voltage applied to the armature or by changing the field current. The introduction of variable resistance in the armature circuit or field circuit allowed speed control. Modern DC motors are often controlled by power electronics  systems called DC drives.

Fig. Motor

Usage

The DC motor or Direct Current Motor to give it its full title, is the most commonly used actuator for producing continuous movement and whose speed of rotation can easily be controlled, making them ideal for use in applications were speed control, servo type control, and/or positioning is required. A DC motor consists of two parts, a "Stator" which is the stationary part and a "Rotor" which is the rotating part. The result is that there are basically three types of DC Motor available.

2.11 GLOBAL SYSTEM FOR MOBILE

Global System for Mobile Communications (GSM) modems are specialized types of modems that operate over subscription based wireless networks, similar to a mobile phone. A GSM modem accepts a Subscriber Identity Module (SIM) card, and basically acts like a mobile phone for a computer. Such a modem can even be a dedicated mobile phone that the computer uses for GSM network capabilities.

Traditional modems are attached to computers to allow dial-up connections to other computer systems. A GSM modem operates in a similar fashion, except that it sends and receives data through radio waves rather than a telephone line. This type of modem may be an external device connected via a Universal Serial Bus (USB) cable or a serial cable. More commonly, however, it is a small device that plugs directly into the USB port or card slot on a computer or laptop.

It is widely used mobile communication system in the world. GSM is an open and digital cellular technology used for transmitting mobile voice and data services operates at the 850MHz, 900MHz, 1800MHz and 1900MHz frequency bands.

#### 2.11.1 Features

- Improved spectrum efficiency

- International roaming

- Compatibility with integrated services digital network (ISDN)

- Support for new services.

- SIM phonebook management

- Fixed dialing number (FDN)

- Real time clock with alarm management

- High-quality speech

- Uses encryption to make phone calls more secure

- Short message service (SMS)

A GSM modem is a wireless modem that works with a GSM wireless network. A wireless modem behaves like a dial-up modem. The main difference between them is that a dial-up modem sends and receives data through a fixed telephone line while a wireless modem sends and receives data through radio waves.

A GSM modem can be an external device or a PC Card / PCMCIA Card. Typically, an external GSM modem is connected to a computer through a serial cable or a USB cable. A GSM modem in the form of a PC Card / PCMCIA Card is designed for use with a laptop computer. It should be inserted into one of the PC Card / PCMCIA Card slots of a laptop computer. Like a GSM mobile phone, a GSM modem requires a SIM card from a wireless carrier in order to operate.

As mentioned in earlier sections of this SMS tutorial, computers use AT commands to control modems. Both GSM modems and dial-up modems support a common set of standard AT commands. You can use a GSM modem just like a dial-up modem.

In addition to the standard AT commands, GSM modems support an extended set of AT commands. These extended AT commands are defined in the GSM standards. With the extended AT commands, you can do things like:

- Reading, writing and deleting SMS messages.

- Sending SMS messages.

- Monitoring the signal strength.

- Monitoring the charging status and charge level of the battery.

- Reading, writing and searching phone book entries.

The number of SMS messages that can be processed by a GSM modem per minute is very low -- only about six to ten SMS messages per minute.

GSM Modem Application

THE GSM NETWORK

GSM provides recommendations, not requirements. The GSM specifications define the functions and interface requirements in detail but do not address the hardware. The reason for this is to limit the designers as little as possible but still to make it possible for the operators to buy equipment from different suppliers. The GSM network is divided into three major systems: the switching system (SS), the base station system (BSS), and the operation and support system (OSS).

GSM Network Elements

The Switching System

The switching system (SS) is responsible for performing call processing and subscriber-related functions. The switching system includes the following functional units.

- Home location register (HLR)—The HLR is a database used for storage and management of subscriptions. The HLR is considered the most important database, as it stores permanent data about subscribers, including a subscriber's service profile, location information, and activity status. When an individual buys a subscription from one of the PCS operators, he or she is registered in the HLR of that operator.

- mobile services switching center (MSC)—The MSC performs the telephony switching functions of the system. It controls calls to and from other telephone and data systems. It also performs such functions as toll ticketing, network interfacing, common channel signaling, and others.

- visitor location register (VLR)—The VLR is a database that contains temporary information about subscribers that is needed by the MSC in order to service visiting subscribers. The VLR is always integrated with the MSC. When a mobile station roams into a new MSC area, the VLR connected to that MSC will request data about the mobile station from the HLR. Later, if the mobile station makes a call, the VLR will have the information needed for call setup without having to interrogate the HLR each time.

- authentication center (AUC)—A unit called the AUC provides authentication and encryption parameters that verify the user's identity and ensure the confidentiality of each call. The AUC protects network operators from different types of fraud found in today's cellular world.

- equipment identity register (EIR)—The EIR is a database that contains information about the identity of mobile equipment that prevents calls from stolen, unauthorized, or defective mobile stations. The AUC and EIR are implemented as stand-alone nodes or as a combined AUC/EIR node.

The Base Station System (BSS)

All radio-related functions are performed in the BSS, which consists of base station controllers (BSCs) and the base transceiver stations (BTSs).

- BSC—The BSC provides all the control functions and physical links between the MSC and BTS. It is a high-capacity switch that provides functions such as handover, cell configuration data, and control of radio frequency (RF) power levels in base transceiver stations. A number of BSCs are served by an MSC.

- BTS—The BTS handles the radio interface to the mobile station. The BTS is the radio equipment (transceivers and antennas) needed to service each cell in the network. A group of BTSs are controlled by a BSC.

The Operation and Support System

The operations and maintenance center (OMC) is connected to all equipment in the switching system and to the BSC. The implementation of OMC is called the operation and support system (OSS). The OSS is the functional entity from which the network operator monitors and controls the system. The purpose of OSS is to offer the customer cost-effective support for centralized, regional, and local operational and maintenance activities that are required for a GSM network. An important function of OSS is to provide a network overview and support the maintenance activities of different operation and maintenance organizations.

GSM NETWORK AREAS

The GSM network is made up of geographic areas. As shown in 4.2, these areas include cells, location areas (LAs), MSC/VLR service areas, and public land mobile network (PLMN) areas.

Network Areas

The cell is the area given radio coverage by one base transceiver station. The GSM network identifies each cell via the cell global identity (CGI) number assigned to each cell. The location area is a group of cells. It is the area in which the subscriber is paged. Each LA is served by one or more base station controllers, yet only by a single MSC (fig 4.3). Each LA is assigned a location area identity (LAI) number.

Location Areas

An MSC/VLR service area represents the part of the GSM network that is covered by one MSC and which is reachable, as it is registered in the VLR of the MSC (fig 4.4).

MSC /VLR Service Areas

The PLMN service area is an area served by one network operator(fig 4.5).

SPECIFICATIONS AND CHARACTERISTICS FOR GSM

- frequency band—The frequency range specified for GSM is 1,850 to 1,990 MHz (mobile station to base station).

- duplex distance—The duplex distance is 80 MHz. Duplex distance is the distance between the uplink and downlink frequencies. A channel has two frequencies, 80 MHz apart.

- channel separation—The separation between adjacent carrier frequencies. In GSM, this is 200 kHz.

- modulation—Modulation is the process of sending a signal by changing the characteristics of a carrier frequency. This is done in GSM via Gaussian minimum shift keying (GMSK).

- transmission rate—GSM is a digital system with an over-the-air bit rate of 270 kbps.

- access method—GSM utilizes the time division multiple access (TDMA) concept. TDMA is a technique in which several different calls may share the same carrier. Each call is assigned a particular time slot.

- speech coder—GSM uses linear predictive coding (LPC). The purpose of LPC is to reduce the bit rate. The LPC provides parameters for a filter that mimics the vocal tract. The signal passes through this filter, leaving behind a residual signal. Speech is encoded at 13 kbps.

GSM SUBSCRIBER SERVICES

There are two basic types of services offered through GSM: telephony (also referred to as teleservices) and data (also referred to as bearer services). Telephony services are mainly voice services that provide subscribers with the complete capability (including necessary terminal equipment) to communicate with other subscribers. Data services provide the capacity necessary to transmit appropriate data signals between two access points creating an interface to the network. In addition to normal telephony and emergency calling, the following subscriber services are supported by GSM:

- dual-tone multifrequency (DTMF)—DTMF is a tone signaling scheme often used for various control purposes via the telephone network, such as remote control of an answering machine. GSM supports full-originating DTMF.

- facsimile group III—GSM supports CCITT Group 3 facsimile. As standard fax machines are designed to be connected to a telephone using analog signals, a special fax converter connected to the exchange is used in the GSM system. This enables a GSM–connected fax to communicate with any analog fax in the network.

- short message services—A convenient facility of the GSM network is the short message service. A message consisting of a maximum of 160 alphanumeric characters can be sent to or from a mobile station. This service can be viewed as an advanced form of alphanumeric paging with a number of advantages. If the subscriber's mobile unit is powered off or has left the coverage area, the message is stored and offered back to the subscriber when the mobile is powered on or has reentered the coverage area of the network. This function ensures that the message will be received.

- cell broadcast—A variation of the short message service is the cell broadcast facility. A message of a maximum of 93 characters can be broadcast to all mobile subscribers in a certain geographic area. Typical applications include traffic congestion warnings and reports on accidents.

- voice mail—This service is actually an answering machine within the network, which is controlled by the subscriber. Calls can be forwarded to the subscriber's voice-mail box and the subscriber checks for messages via a personal security code.

fax mail—With this service, the subscriber can receive fax messages at any fax machine. The messages are stored in a service center from which they can be retrieved by the subscriber via a personal security code to the desired fax number.

CHAPTER 3

SOFTWARE SPECIFICATION

KEIL SOFTWARE

Keil compiler is a software used where the machine language code is written and compiled. After compilation, the machine source code is converted into hex code which is to be dumped into the microcontroller for further processing. Keil compiler also supports C language code.

General Introduction

Keil Software is the leading vendor for 8/16-bit development tools (ranked at first position in the 2004 Embedded Market Study of the Embedded Systems and EE Times magazine). Keil Software is represented world-wide in more than 40 countries. Since the market introduction in 1988, the Keil C51 Compiler is the de facto industry standard and supports more than 500 current 8051 device variants. Now, Keil Software offers development tools for ARM.

Keil Software makes C compilers, macro assemblers, real-time kernels, debuggers, simulators, integrated environments, and evaluation boards for the 8051, 251, ARM, and XC16x/C16x/ST10 microcontroller families.

Keil Software is pleased to announce simulation support for the Atmel AT91 ARM family of microcontrollers. The Keil µVision Debugger simulates the complete ARM instruction-set as well as the on-chip peripherals for each device in the AT91 ARM/Thumb microcontroller family. The integrated simulator provides complete peripheral simulation. Other new features in the µVision Debugger include:

- An integrated Software Logic Analyzer that measures I/O signals as well as program variables and helps developers create complex signal processing algorithms.

- An Execution Profiler that measures time spent in each function, source line, and assembler instruction. Now developers can find exactly where programs spend the most time.

"Using nothing more than the provided simulation support and debug scripts, developers can create a high-fidelity simulation of their actual target hardware and environment. No extra hardware or test equipment is required. The Logic Analyzer and Execution Profiler will help developers when it comes time to develop and tune signaling algorithms." said Jon Ward, President of Keil Software USA, Inc.

# µVision3 Overview

The µVision3 IDE is a Windows-based software development platform that combines a robust editor, project manager, and makes facility. µVision3 integrates all tools including the C compiler, macro assembler, linker/locator, and HEX file generator. µVision3 helps expedite the development process of your embedded applications by providing the following:

- Full-featured source code editor,

- Device database for configuring the development tool setting,

- Project manager for creating and maintaining your projects,

- Integrated make facility for assembling, compiling, and linking your embedded applications,

- Dialogs for all development tool settings,

- True integrated source-level Debugger with high-speed CPU and peripheral simulator,

- Advanced GDI interface for software debugging in the target hardware and for connection to Keil ULINK,

- Flash programming utility for downloading the application program into Flash ROM,

- Links to development tools manuals, device datasheets & user’s guides.

The µVision3 IDE offers numerous features and advantages that help you quickly and successfully develop embedded applications. They are easy to use and are guaranteed to help you achieve your design goals.

The µVision3 IDE and Debugger is the central part of the Keil development tool chain. µVision3 offers a Build Mode and a Debug Mode.

In the µVision3 Build Mode you maintain the project files and generate the application.

In the µVision3 Debug Mode you verify your program either with a powerful CPU and peripheral simulator or with the Keil ULINK USB-JTAG Adapter (or other AGDI drivers) that connect the debugger to the target system. The ULINK allows you also to download your application into Flash ROM of your target system.

STEPS TO WRITE AN ASSEMBLY LANGUAGE PROGRAM IN KEIL AND HOW TO COMPILE IT:

- Install the Keil Software in the PC in any of the drives.

- After installation, an icon will be created with the name “Keil uVision3”. Just drag this icon onto the desktop so that it becomes easy whenever you try to write programs in keil.

- Double click on this icon to start the keil compiler.

- A page opens with different options in it showing the project workspace at the leftmost corner side, output window in the bottom and an ash coloured space for the program to be written.

- Now to start using the keil, click on the option “project”.

- A small window opens showing the options like new project, import project, open project etc. Click on “New project”.

- A small window with the title bar “Create new project” opens. The window asks the user to give the project name with which it should be created and the destination location. The project can be created in any of the drives available. You can create a new folder and then a new file or can create directly a new file.

- After the file is saved in the given destination location, a window opens where a list of vendors will be displayed and you have to select the device for the target you have created.

- The most widely used vendor is Atmel. So click on Atmel and now the family of microcontrollers manufactured by Atmel opens. You can select any one of the microcontrollers according to the requirement.

- When you click on any one of the microcontrollers, the features of that particular microcontroller will be displayed on the right side of the page. The most appropriate microcontroller with which most of the projects can be implemented is the AT89C51. Click on this microcontroller and have a look at its features. Now click on “OK” to select this microcontroller.

- A small window opens asking whether to copy the startup code into the file you have created just now. Just click on “No” to proceed further.

- Now you can see the TARGET and SOURCE GROUP created in the project workspace.

- Now click on “File” and in that “New”. A new page opens and you can start writing program in it.

- After the program is completed, save it with any name but with the .asm extension. Save the program in the file you have created earlier.

- You can notice that after you save the program, the predefined keywords will be highlighted in bold letters.

- Now add this file to the target by giving a right click on the source group. A list of options open and in that select “Add files to the source group”. Check for this file where you have saved and add it.

- Right click on the target and select the first option “Options for target”. A window opens with different options like device, target, output etc. First click on “target”.

- Since the set frequency of the microcontroller is 11.0592 MHz to interface with the PC, just enter this frequency value in the Xtal (MHz) text area and put a tick on the Use on-chip ROM. This is because the program what we write here in the keil will later be dumped into the microcontroller and will be stored in the inbuilt ROM in the microcontroller.

- Now click the option “Output” and give any name to the hex file to be created in the “Name of executable” text area and put a tick to the “Create HEX file” option present in the same window. The hex file can be created in any of the drives. You can change the folder by clicking on “Select folder for Objects”.

- Now to check whether the program you have written is errorless or not, click on the icon exactly below the “Open file” icon which is nothing but Build Target icon. You can even use the shortcut key F7 to compile the program written.

- To check for the output, there are several windows like serial window, memory window, project window etc. Depending on the program you have written, select the appropriate window to see the output by entering into debug mode.

- The icon with the letter “d” indicates the debug mode.

- Click on this icon and now click on the option “View” and select the appropriate window to check for the output.

- After this is done, click the icon “debug” again to come out of the debug mode.

- The hex file created as shown earlier will be dumped into the microcontroller with the help of another software called Proload/Topwin.

CHAPTER 4

IMPLEMENTATION

4.1 SCHEMATIC DIAGRAM:

CHAPTER 5

SIMULATION AND DESIGN

5.1 OUTPUT SCREEN SHOTS

CHAPTER 6

CONCLUSION

A lucrative environmental monitoring method with least amount of components has been constructed. The system is successfully developed using wired and wireless networks. The limitations of wired network and opportunities using wireless networks are rigorously described. The environment monitoring sensors with an microcontroller, GSM is proposed. For sending and receiving of the data.. We envisage that this system will encompass an enormous recognition in the industrialized sectors and will realize an effective amalgamation among WSN. Accordingly, a tack target of inaccessible monitoring of the air quality within the environment can be attained. It is highly pertinent to metrological departments and also in industrial sectors. In future, we would be fond of to be made controlling system for environment monitoring.

REFERENCES:

- “Ambient (outdoor) air quality and health,” September 2016, http://www.who.int/mediacentre/factsheets/fs313/en/.View at: Google Scholar

- K. Vijayaraghavan, “Green roofs : a critical review on the role of components, benefits, limitations and trends,” Renewable and Sustainable Energy Reviews, vol. 57, pp. 740–752, 2016.View at: Publisher Site | Google Scholar

- D. B. Rowe, “Green roofs as a means of pollution abatement,” Environmental Pollution, vol. 159, no. 8-9, pp. 2100–2110, 2011.View at: Publisher Site | Google Scholar

- T. Kim, A. Saini, J. Kim et al., “Piezoelectric floating element shear stress sensor for the wind tunnel flow measurement,” IEEE Transactions on Industrial Electronics, vol. 64, no. 9, pp. 7304–7312, 2017.View at: Publisher Site | Google Scholar

- P. Spachos and D. Hatzinakos, “Real-time indoor carbon dioxide monitoring through cognitive wireless sensor networks,” IEEE Sensors Journal, vol. 16, no. 2, pp. 506–514, 2016.View at: Publisher Site | Google Scholar